# Reproducibility Guide

## MASS — Multi-Agent Cyber-Physical Security System

Team Members: Hla Ahmed, Arwa Ahmed, Menna Salem, Mena Osman, Malak Amgad

## Table of Contents

Project Overview

System Architecture

Prerequisites — All Team Members

Part 1 — Server Room Setup (Menna)

Part 2 — HQ VM Setup (Arwa)

Part 3 — Mac Agents Setup (Hala)

Part 4 — Raspberry Pi PAC Setup (Malak)

Verification Checklist

Demo Attack Scenario

System Reset Procedure

Repository Structure

Quick Reference Tables

Troubleshooting

## 1. Project Overview

MASS (Multi-Agent Security System) is a distributed, multi-agent cyber-physical security platform designed to protect a campus network. It unifies physical access control, IoT device monitoring, and data network intrusion detection under a single coordinated intelligence layer. The system is composed of 18 autonomous agents running across five machines, all communicating through Apache Kafka as the central message bus.

### 1.1 Design Goals

| Goal | Description |
| --- | --- |
| Distributed Detection | Independent agents monitor IoT, physical access, and data networks simultaneously |
| Cross-Domain Correlation | An analytical agent fuses alerts from all domains to detect coordinated attacks |
| Automated Response | A SOAR orchestrator triggers playbooks with human-in-the-loop approval for high-impact actions |
| Continuous Learning | A learning agent retrains anomaly-detection models from production telemetry |
| Real-Time Visibility | A 17-page SOC dashboard provides live threat overview, kill-chain analysis, SOAR tracking, and forensic collection |

### 1.2 Technology Stack

| Layer | Technology | Version |
| --- | --- | --- |
| Message Broker | Apache Kafka (Confluent) | 7.5.0 |
| Database (Documents) | MongoDB | 7.0 |
| Database (Time Series) | InfluxDB | 2.7 |
| Database (Relational) | PostgreSQL | 16 |
| MQTT Broker | Eclipse Mosquitto | 2.x |
| Directory Service | OpenLDAP | Latest |
| Container Runtime | Docker + Docker Compose | Latest |
| Agent Runtime | Python | 3.11 / 3.12 |
| Dashboard Frontend | React (standalone) | 18.3.1 |
| Dashboard Transpiler | Babel (standalone) | 7.29.0 |
| Monitoring | Grafana | 10.3.1 |
| Physical Access | MFRC522 RFID + OpenCV Face Recognition | — |
| Network Detection | Zeek IDS + Suricata | Latest |

## 2. System Architecture

┌─────────────────────────────────────────────────────────────────────┐
│                        MASS Architecture                            │
│                                                                     │
│   ┌──────────────────────┐         ┌──────────────────────┐        │
│   │   SERVER ROOM        │         │   HQ VM              │        │
│   │   (Menna)            │         │   (Arwa)             │        │
│   │   192.168.60.x       │         │   192.168.12.10      │        │
│   │                      │         │                      │        │
│   │   • Kafka 9092       │◀═══════▶│   • Analytical 8006  │        │
│   │   • MongoDB 27017    │         │   • Orchestrator 8007│        │
│   │   • MQTT 1883        │         │   • Learning 8008    │        │
│   │   • LDAP 389         │         │   • TI Agent 8009    │        │
│   │   • Gateway Agent    │         │   • Central Mgr 8020 │        │
│   │   • Behavioral Agent │         │   • Forensic 8021    │        │
│   │   • NDR/EDR Agents   │         │   • Dashboard 8080   │        │
│   │   • IoT/PAC/Data LMs │         │   • Heartbeat Monit. │        │
│   └──────────────────────┘         └──────────────────────┘        │
│              ▲                                ▲                     │
│              │          Kafka Bus             │                     │
│              │      agents.heartbeats         │                     │
│              │      hq.incidents              │                     │
│              │      hq.correlated             │                     │
│              │      soar.commands             │                     │
│              ▼                                ▼                     │
│   ┌──────────────────────┐         ┌──────────────────────┐        │
│   │   MAC AGENTS         │         │   RASPBERRY PI       │        │
│   │   (Hala)             │         │   (Malak)            │        │
│   │   MAC: 0c:3d:5e:01:  │         │   172.20.10.4        │        │
│   │        bf:db          │         │                      │        │
│   │                      │         │   • RFID RC522        │        │
│   │   • NDR Agent        │         │   • USB Camera        │        │
│   │   • EDR Agent        │         │   • Face Recognition  │        │
│   │   • Zeek IDS         │         │   • Door Controller   │        │
│   │   • Suricata         │         │                      │        │
│   └──────────────────────┘         └──────────────────────┘        │
└─────────────────────────────────────────────────────────────────────┘

### 2.1 Agent Inventory

| # | Agent ID | Machine | Port | Domain |
| --- | --- | --- | --- | --- |
| 1 | analytical-agent-01 | HQ VM | 8006 | HQ |
| 2 | orchestrator-agent-01 | HQ VM | 8007 | HQ |
| 3 | learning-agent-01 | HQ VM | 8008 | HQ |
| 4 | threat-intel-agent-01 | HQ VM | 8009 | HQ |
| 5 | central-manager-01 | HQ VM | 8020 | HQ |
| 6 | forensic-agent-hq-01 | HQ VM | 8021 | HQ |
| 7 | gateway-agent-01 | Server Room | 8000 | IoT |
| 8 | behavioral-agent-01 | Server Room | 8001 | IoT |
| 9 | iot-local-manager-01 | Server Room | 8010 | IoT |
| 10 | pac-eda-agent-01 | Server Room | 8002 | PAC |
| 11 | credential-anomaly-agent-01 | Server Room | 8003 | PAC |
| 12 | pac-local-manager-01 | Server Room | 8011 | PAC |
| 13 | ndr-agent-01 | Server Room | 8004 | Data |
| 14 | edr-agent-01 | Server Room | 8005 | Data |
| 15 | data-local-manager-01 | Server Room | 8012 | Data |
| 16 | fire_system_agent | Server Room | — | IoT |
| 17 | ndr-agent-0c-3d-5e-01-bf-db | Mac | — | Data |
| 18 | edr-agent-0c-3d-5e-01-bf-db | Mac | — | Data |

### 2.2 Data Flow Summary

Sensors/Devices
     │
     ▼
  Field Agents (NDR, EDR, Gateway, PAC-EDA, Behavioral)
     │  publish to: iot.alerts, data.alerts, pac.alerts
     ▼
  Local Managers (IoT-LM, PAC-LM, Data-LM)
     │  correlate locally, escalate to: hq.incidents
     ▼
  Central Manager
     │  aggregates all incidents, calculates threat level
     ▼
  Analytical Agent
     │  cross-domain correlation → hq.correlated
     ▼
  Orchestrator Agent
     │  triggers SOAR playbooks → soar.commands
     │  waits for approval → soar.responses
     ▼
  Dashboard (SOC Enterprise)
     │  real-time visualization of all layers
     ▼
  Forensic Agent → collects evidence bundles
  TI Agent → enriches IOCs with threat intelligence
  Learning Agent → retrains models periodically

## 3. Prerequisites — All Team Members

### 3.1 Network Requirements

All machines must be connected to the same campus network (university ethernet) and able to reach the following services:

| Service | Address | Port | Protocol |
| --- | --- | --- | --- |
| Kafka Broker | 192.168.60.10 | 9092 | TCP |
| MongoDB | 192.168.60.12 | 27017 | TCP |
| MQTT Broker | 192.168.60.10 | 1883 | TCP |
| LDAP Server | 192.168.60.11 | 389 | TCP |
| HQ VM | 192.168.12.10 | 8080 | HTTP |
| Core Switch | 192.168.40.1 | — | ICMP |

Verify connectivity before starting any component:

# From every machine, run:
ping -c 3 192.168.60.10   # Kafka broker
ping -c 3 192.168.12.10   # HQ VM

### 3.2 Required Startup Order

CRITICAL: The system must start in this exact sequence. Starting out of order will cause Kafka connection failures, missing heartbeats, and orphaned agents.

| Step | Action | Wait Before Next |
| --- | --- | --- |
| 1 | Start Server Room (Kafka + MongoDB + MQTT + Local Managers) | Until Kafka is accepting connections |
| 2 | Start HQ VM (Central Manager + HQ Agents + Dashboard) | 120 seconds (grace period) |
| 3 | Start Mac Agents (NDR + EDR) | Until heartbeats appear |
| 4 | Start PAC Pi (RFID + door controller) | Until PAC events visible |
| 5 | Verify all 18 agents alive on dashboard | — |

### 3.3 Common Software (All Machines)

Every machine running MASS agents requires:

# Python 3.11 or 3.12
python3 --version   # must show 3.11.x or 3.12.x

# confluent-kafka Python client
pip install confluent-kafka

# FastAPI + Uvicorn (for /health endpoints)
pip install fastapi uvicorn

## 4. Part 1 — Server Room

### 4.1 Machine Specifications

| Property | Value |
| --- | --- |
| OS | Ubuntu Server |
| IP Addresses | 192.168.60.10 (Kafka/MQTT), 192.168.60.11 (LDAP), 192.168.60.12 (MongoDB) |
| Role | Kafka broker, MongoDB, MQTT, LDAP, Local Managers, Building agents |

### 4.2 Software Installation

#### 4.2.1 Apache Kafka + Zookeeper

# Download Kafka (Confluent Platform recommended)
wget https://downloads.apache.org/kafka/3.6.0/kafka_2.13-3.6.0.tgz
tar -xzf kafka_2.13-3.6.0.tgz
cd kafka_2.13-3.6.0

#### 4.2.2 MongoDB

sudo apt update
sudo apt install -y mongodb
sudo systemctl enable mongodb
sudo systemctl start mongodb

#### 4.2.3 MQTT Broker (Mosquitto)

sudo apt install -y mosquitto mosquitto-clients

#### 4.2.4 OpenLDAP

sudo apt install -y slapd ldap-utils

#### 4.2.5 Python Packages

pip install confluent-kafka fastapi uvicorn paho-mqtt pymongo

### 4.3 Kafka Configuration

Edit config/server.properties inside the Kafka directory:

# Broker identity
broker.id=0

# Listen on all interfaces, advertise the campus IP
listeners=PLAINTEXT://0.0.0.0:9092
advertised.listeners=PLAINTEXT://192.168.60.10:9092

# Log storage
log.dirs=/tmp/kafka-logs

# Zookeeper connection
zookeeper.connect=localhost:2181

# Auto-create topics (enabled for convenience)
auto.create.topics.enable=true

# Retention: 7 days
log.retention.hours=168

### 4.4 Start Kafka

# 1. Start Zookeeper (must come first)
bin/zookeeper-server-start.sh config/zookeeper.properties &
sleep 5

# 2. Start Kafka broker
bin/kafka-server-start.sh config/server.properties &
sleep 10

# 3. Verify broker is running
bin/kafka-broker-api-versions.sh --bootstrap-server 192.168.60.10:9092

### 4.5 Create Kafka Topics

The system requires 17 topics. Create them with the following script:

#!/bin/bash
# create_topics.sh — Run once after Kafka starts

BOOTSTRAP="192.168.60.10:9092"

topics=(
  "agents.heartbeats"
  "hq.incidents"
  "hq.correlated"
  "soar.commands"
  "soar.responses"
  "data.alerts"
  "data.incidents"
  "iot.alerts"
  "iot.incidents"
  "iot.telemetry"
  "pac.alerts"
  "pac.events"
  "pac.incidents"
  "pac.snapshots"
  "learning.model_updates"
  "ti.enriched"
  "forensic.bundles"
)

for topic in "${topics[@]}"; do
  bin/kafka-topics.sh --create \
    --bootstrap-server "$BOOTSTRAP" \
    --topic "$topic" \
    --partitions 3 \
    --replication-factor 1 \
    --if-not-exists
  echo "Created: $topic"
done

echo ""
echo "=== Verification ==="
bin/kafka-topics.sh --list --bootstrap-server "$BOOTSTRAP"

### 4.6 MQTT Configuration

Edit /etc/mosquitto/mosquitto.conf:

# Allow connections from all interfaces
listener 1883 0.0.0.0

# Allow anonymous connections (campus-internal only)
allow_anonymous true

# Logging
log_type all
log_dest file /var/log/mosquitto/mosquitto.log

Start and enable:

sudo systemctl start mosquitto
sudo systemctl enable mosquitto

# Verify
mosquitto_pub -h localhost -t "test/topic" -m "hello"
mosquitto_sub -h localhost -t "test/topic" -C 1

### 4.7 MongoDB Setup

sudo systemctl start mongodb

# Create the MASS database and initial collections
mongo --eval "
  db = db.getSiblingDB('mass_db');
  db.createCollection('incidents');
  db.createCollection('correlations');
  db.createCollection('forensic_bundles');
  db.createCollection('agent_health');
  print('MASS database initialized');
"

### 4.8 LDAP Setup

sudo dpkg-reconfigure slapd
# Set domain: dc=mass,dc=campus
# Set organization: MASS Campus Security
# Set admin password: <your-password>

# Load team member entries
ldapadd -x -D "cn=admin,dc=mass,dc=campus" -W -f team_ldap_entries.ldif

Example `team_ldap_entries.ldif`:

dn: ou=users,dc=mass,dc=campus
objectClass: organizationalUnit
ou: users

dn: uid=arwa,ou=users,dc=mass,dc=campus
objectClass: inetOrgPerson
cn: Arwa
sn: Arwa
uid: arwa
rfidCard: <CARD_UID_HEX>

dn: uid=hala,ou=users,dc=mass,dc=campus
objectClass: inetOrgPerson
cn: Hala
sn: Hala
uid: hala
rfidCard: <CARD_UID_HEX>

dn: uid=malak,ou=users,dc=mass,dc=campus
objectClass: inetOrgPerson
cn: Malak
sn: Malak
uid: malak
rfidCard: <CARD_UID_HEX>

dn: uid=menna,ou=users,dc=mass,dc=campus
objectClass: inetOrgPerson
cn: Menna
sn: Menna
uid: menna
rfidCard: <CARD_UID_HEX>

### 4.9 Start Building Agents

All building-level agents (IoT, PAC, Data) run as Docker containers on the server room machine:

cd ~/mass-server-room
./start_building_agents.sh

Or manually start the Docker Compose stack (if using the unified compose file):

cd ~/agents-test-main/docker
docker-compose up -d \
  gateway-agent \
  behavioral-agent \
  ndr-agent \
  edr-agent \
  pac-eda-agent \
  credential-anomaly-agent \
  iot-local-manager \
  pac-local-manager \
  data-local-manager

### 4.10 Verify Server Room Agents

# Verify agents are sending heartbeats to Kafka
python3 -c "
from confluent_kafka import Consumer
import json, time

c = Consumer({
    'bootstrap.servers': '192.168.60.10:9092',
    'group.id': 'verify-' + str(int(time.time())),
    'auto.offset.reset': 'latest'
})
c.subscribe(['agents.heartbeats'])

start = time.time()
seen = set()
while time.time() - start < 30:
    msg = c.poll(1.0)
    if msg and not msg.error():
        d = json.loads(msg.value().decode())
        seen.add(d.get('agent_id'))

c.close()
print('Agents found:', sorted(seen))
"

Expected agents from Server Room:

gateway-agent-01

behavioral-agent-01

ndr-agent-01

edr-agent-01

pac-eda-agent-01

credential-anomaly-agent-01

iot-local-manager-01

pac-local-manager-01

data-local-manager-01

fire_system_agent

## 5. Part 2 — HQ VM Setup

### 5.1 Machine Specifications

| Property | Value |
| --- | --- |
| OS | Ubuntu 24.04 LTS |
| Hypervisor | VMware Workstation 16 Pro |
| IP | 192.168.12.10 (static) |
| VLAN | 12 |
| Role | Central Manager, HQ Agents, SOC Dashboard |

### 5.2 Software Installation

#### 5.2.1 Docker

sudo apt update
sudo apt install -y docker.io docker-compose
sudo usermod -aG docker $USER
# Log out and back in for group change to take effect

#### 5.2.2 Python

sudo apt install -y python3.11 python3-pip

#### 5.2.3 Python Packages

pip install confluent-kafka fastapi uvicorn paramiko requests pydantic

Note (offline VM): If the VM has no internet access, install packages on a machine with internet, then copy the wheel files to the VM via USB or scp.

### 5.3 Static IP Configuration

Edit /etc/netplan/50-cloud-init.yaml:

network:
  ethernets:
    ens33:
      addresses: [192.168.12.10/24]
      gateway4: 192.168.12.1
      nameservers:
        addresses: [8.8.8.8]
  version: 2

Apply and verify:

sudo netplan apply
ip addr show | grep 192.168.12.10

### 5.4 Repository Setup

# Clone the main HQ repository
git clone https://github.com/<YOUR_USERNAME>/mass-hq.git ~/agents-test-main
cd ~/agents-test-main

# Clone standalone agents (separate repos)
git clone https://github.com/<YOUR_USERNAME>/mass-ti-agent.git ~/ti_agent
git clone https://github.com/<YOUR_USERNAME>/mass-forensic-agent.git ~/forensic_agent

### 5.5 Frontend Dependencies (Local JS Libraries)

The dashboard uses standalone React and Babel loaded from local files (no CDN or npm required). If you are setting up the VM without internet, download these files on another machine first:

| File | Version | Download URL | Place In |
| --- | --- | --- | --- |
| react.development.js | 18.3.1 | https://unpkg.com/react@18.3.1/umd/react.development.js | ~/agents-test-main/src/ |
| react-dom.development.js | 18.3.1 | https://unpkg.com/react-dom@18.3.1/umd/react-dom.development.js | ~/agents-test-main/src/ |
| babel.min.js | 7.29.0 | https://unpkg.com/@babel/standalone@7.29.0/babel.min.js | ~/agents-test-main/src/ |

Transfer to VM:

# From a machine with internet:
scp react.development.js react-dom.development.js babel.min.js \
    arwa@192.168.12.10:~/agents-test-main/src/

### 5.6 Environment Variables

Create ~/agents-test-main/docker/.env:

# MASS — Environment Variables
# NEVER commit this file to version control

# Kafka
KAFKA_BOOTSTRAP=192.168.60.10:9092

# MongoDB
MONGO_USER=massadmin
MONGO_PASSWORD=mass2026
MONGO_URI=mongodb://192.168.60.10:27017

# InfluxDB
INFLUX_USER=massadmin
INFLUX_PASSWORD=mass2026secure
INFLUX_TOKEN=mass2026tokenchangeme

# PostgreSQL
POSTGRES_USER=massadmin
POSTGRES_PASSWORD=mass2026

# Grafana
GRAFANA_USER=admin
GRAFANA_PASSWORD=mass2026

### 5.7 Build Docker Images

cd ~/agents-test-main/docker

# Build all agent images (uses Dockerfile.agent)
docker-compose build

# Verify images were created
docker images | grep campus

Expected images:

docker_analytical-agent
docker_orchestrator-agent
docker_learning-agent
docker_central-manager

Dockerfile details: All agents share a single Dockerfile.agent based on python:3.12-slim. It installs system dependencies (gcc, libssl-dev, curl, netcat), copies requirements.txt, installs Python packages, copies the full project source, and creates a non-root user (massagent) for security compliance (NIST SP 800-190).

### 5.8 Start the HQ System

Use the provided startup script:

cd ~/agents-test-main
chmod +x start_hq.sh
./start_hq.sh

What `start_hq.sh` does (in order):

| Step | Action | Port |
| --- | --- | --- |
| 1 | Start analytical-agent via docker-compose | 8006 |
| 2 | Start orchestrator-agent via docker-compose | 8007 |
| 3 | Start learning-agent via docker-compose | 8008 |
| 4 | Start central-manager via docker-compose | 8020 |
| 5 | Deploy latest code via docker cp + restart containers | — |
| 6 | Start TI Agent as Python process | 8009 |
| 7 | Start Forensic Agent as Python process | 8021 |
| 8 | Start Heartbeat Monitor in background | — |
| 9 | Start Dashboard Proxy Server | 8080 |

Alternatively, use the full startup script (start_all.sh) which also auto-detects Kafka connectivity:

cd ~/mass_hq_full_20260614_2144
chmod +x start_all.sh
./start_all.sh

### 5.9 Verify HQ is Running

# Check all 6 agent/service ports
for port in 8006 8007 8008 8009 8020 8021; do
  status=$(curl -s --max-time 2 http://localhost:$port/health | \
    python3 -c "import sys,json; print(json.load(sys.stdin).get('status','?'))" \
    2>/dev/null || echo "DOWN")
  echo "Port $port: $status"
done

Expected output:

Port 8006: running    ← analytical-agent
Port 8007: running    ← orchestrator-agent
Port 8008: running    ← learning-agent
Port 8009: running    ← ti-agent
Port 8020: running    ← central-manager
Port 8021: running    ← forensic-agent

Open the dashboard:

http://192.168.12.10:8080/soc_enterprise.html

### 5.10 Grace Period (Important)

IMPORTANT: Wait 2 minutes (120 seconds) after startup before using the dashboard. The Central Manager has a built-in grace period to prevent Kafka's historical backlog from triggering false alerts and flooding the pending approvals queue.

After the grace period:

sleep 120

# Clear any backlog that accumulated during startup
curl -s -X POST http://localhost:8007/clear_pending
curl -s -X POST http://localhost:8007/clear_executions

### 5.11 Dashboard Proxy Architecture

The dashboard proxy (server.py) runs on port 8080 and serves two functions:

Static file server — serves HTML, JSX, JS, and CSS files from the project directory

API reverse proxy — forwards /api/<port>/* requests to the corresponding backend agent, eliminating CORS issues

| Proxy Route | Backend |
| --- | --- |
| /api/8020/* | Central Manager (localhost:8020) |
| /api/8006/* | Analytical Agent (localhost:8006) |
| /api/8007/* | Orchestrator (localhost:8007) |
| /api/8008/* | Learning Agent (localhost:8008) |
| /api/8009/* | TI Agent (localhost:8009) |
| /api/8021/* | Forensic Agent (localhost:8021) |

## 6. Part 3 — Mac Agents Setup

### 6.1 Machine Specifications

| Property | Value |
| --- | --- |
| OS | macOS |
| MAC Address | 0c:3d:5e:01:bf:db |
| VLAN | 10 (student network) or 40 (local manager network) |
| Role | NDR agent, EDR agent |

### 6.2 Software Installation

# Python 3.11
brew install python@3.11

# Required Python packages
pip3 install confluent-kafka fastapi uvicorn scapy

# Zeek IDS (for NDR network analysis)
brew install zeek

# Suricata (for NDR signature-based detection)
brew install suricata

### 6.3 Agent Configuration

Edit the agent configuration files to point to the campus Kafka broker:

# In both NDR and EDR agent config:
KAFKA_BOOTSTRAP = "192.168.60.10:9092"

# NDR agent identity
AGENT_ID = "ndr-agent-0c-3d-5e-01-bf-db"

# EDR agent identity (separate config)
AGENT_ID = "edr-agent-0c-3d-5e-01-bf-db"

### 6.4 Start Agents

# Start NDR agent
cd ~/mass-agents/ndr_agent
python3 main.py &

# Start EDR agent
cd ~/mass-agents/edr_agent
python3 main.py &

### 6.5 Verification

Check the HQ dashboard:

http://192.168.12.10:8080/soc_enterprise.html

Navigate to the Overview page → Honeycomb Grid. The following agents should show as LIVE (green):

ndr-agent-0c-3d-5e-01-bf-db

edr-agent-0c-3d-5e-01-bf-db

## 7. Part 4 — Raspberry Pi PAC Setup

### 7.1 Hardware Requirements

| Component | Details |
| --- | --- |
| Board | Raspberry Pi 5 (8GB RAM) |
| OS | Raspberry Pi OS (64-bit) |
| IP | 172.20.10.4 or 10.180.1.3 |
| SSH | ssh pi@172.20.10.4 |
| Role | RFID reader, face recognition, door controller |

### 7.2 Hardware Wiring (RFID Reader RC522)

RC522 Pin    →    Raspberry Pi GPIO
─────────────────────────────────────
SDA          →    GPIO 8   (CE0)
SCK          →    GPIO 11  (SCLK)
MOSI         →    GPIO 10  (MOSI)
MISO         →    GPIO 9   (MISO)
IRQ          →    Not connected
GND          →    GND
RST          →    GPIO 25
3.3V         →    3.3V

Camera: USB camera module or Raspberry Pi Camera Module connected via CSI ribbon cable.

### 7.3 Software Installation

# System packages
sudo apt update
sudo apt install -y python3-pip python3-opencv libatlas-base-dev

# Create virtual environment
python3 -m venv ~/myenv
source ~/myenv/bin/activate

# Install Python packages in virtual environment
pip install confluent-kafka mfrc522 requests face_recognition

CRITICAL — GPIO Conflict Fix:
``bash
# Remove the conflicting RPi.GPIO library
pip uninstall RPi.GPIO

# Install the rpi-lgpio shim instead (required for Pi 5)
pip install rpi-lgpio
``

CRITICAL — Camera Compatibility:
The camera must use system Python (not the virtual environment). OpenCV 4.13.0 under system Python works correctly; OpenCV 4.8.0 in the virtual environment does NOT work for camera capture.

### 7.4 Face Enrollment

Must run with system Python (not myenv):

# Deactivate virtual environment if active
deactivate

# Enroll team members' faces
python3 /home/pi/enroll_faces.py

Enrolled users are stored in ~/mass-pi/enrolled_faces.json. Current enrollments: Arwa, Hala, M.O., M.S., Malak.

### 7.5 LDAP Card Registration

On Menna's server, register RFID card UIDs in LDAP:

ldapadd -x -D "cn=admin,dc=mass,dc=campus" -W << 'EOF'
dn: uid=arwa,ou=users,dc=mass,dc=campus
objectClass: inetOrgPerson
uid: arwa
rfidCard: <YOUR_CARD_UID_HEX>
EOF

### 7.6 Start PAC System

# SSH into the Pi
ssh pi@172.20.10.4

# Navigate to project
cd ~/mass-pi

# Activate virtual environment (for confluent-kafka)
source ~/myenv/bin/activate

# Set required environment variables
export LG_CHIP=4                # Required for rpi-lgpio on Pi 5
export QT_QPA_PLATFORM=xcb     # Required for OpenCV GUI
export DISPLAY=:0               # Required for display output

# Start the PAC system
python3 /home/pi/mass_pac_final.py

### 7.7 Verify PAC is Working

Test the full physical access flow:

| Step | Expected Behavior |
| --- | --- |
| 1. Swipe an enrolled RFID card | System reads card UID → checks LDAP |
| 2. LDAP returns authorized | Camera activates → captures face |
| 3. Face matches enrolled user | Access granted → event published to pac.events |
| 4. Check HQ Dashboard | Incidents page shows PAC events |
| 5. Physical Access integrity bar | Changes from IDLE to HEALTHY |

## 8. Verification Checklist

After all four subsystems are started, run through this checklist:

### 8.1 Agent Health — Dashboard

Navigate to: Overview page → Honeycomb Grid

Expected: 14–18 agents showing LIVE (green)

| Agent | Expected Status |
| --- | --- |
| analytical-agent-01 | ✅ LIVE |
| orchestrator-agent-01 | ✅ LIVE |
| learning-agent-01 | ✅ LIVE |
| central-manager-01 | ✅ LIVE |
| threat-intel-agent-01 | ✅ LIVE |
| forensic-agent-hq-01 | ✅ LIVE |
| iot-local-manager-01 | ✅ LIVE |
| pac-local-manager-01 | ✅ LIVE |
| data-local-manager-01 | ✅ LIVE |
| ndr-agent-01 | ✅ LIVE |
| edr-agent-01 | ✅ LIVE |
| gateway-agent-01 | ✅ LIVE |
| behavioral-agent-01 | ✅ LIVE |
| pac-eda-agent-01 | ✅ LIVE |
| credential-anomaly-agent-01 | ✅ LIVE |
| ndr-agent-0c-3d-5e-01-bf-db | ✅ LIVE |
| edr-agent-0c-3d-5e-01-bf-db | ✅ LIVE |

### 8.2 Kafka Health

# From any machine with confluent-kafka installed
python3 -c "
from confluent_kafka.admin import AdminClient

admin = AdminClient({'bootstrap.servers': '192.168.60.10:9092'})
topics = admin.list_topics().topics

print(f'Topics found: {len(topics)}')
for t in sorted(topics.keys()):
    if not t.startswith('_'):  # skip internal topics
        print(f'  {t}')
"

Expected: 17+ topics listed.

### 8.3 End-to-End Flow Test

Inject a synthetic test incident to verify the full pipeline:

python3 -c "
from confluent_kafka import Producer
import json, time

p = Producer({'bootstrap.servers': '192.168.60.10:9092'})

incident = {
    'incident_id': 'TEST-001',
    'alert_type': 'ransomware_behavior',
    'severity': 'CRITICAL',
    'network_domain': 'data_network',
    'agent_type': 'edr',
    'host_id': 'test-host',
    'manager_id': 'data-local-manager-01',
    'created_at': time.strftime('%Y-%m-%dT%H:%M:%S+00:00')
}

p.produce('hq.incidents', json.dumps(incident).encode())
p.flush()
print('Test incident published to hq.incidents')
"

Verify on Dashboard:

Navigate to the Overview page

The test incident should appear in the Recent Incidents table within 5 seconds

The threat level should update accordingly

## 9. Demo Attack Scenario

### 9.1 Pre-Demo Setup

# 1. Clear any leftover state
curl -s -X POST http://localhost:8007/clear_pending
curl -s -X POST http://localhost:8007/clear_executions

# 2. Verify all agents alive on dashboard (AGENTS should show 14+/18)

# 3. Note current counts for comparison
curl -s http://localhost:8020/status | python3 -c "
import sys, json
d = json.load(sys.stdin)
print('Incidents:', d['incidents']['total'])
print('Correlations:', d['correlations_active'])
print('Commands:', d['commands_issued'])
"

### 9.2 Attack Timeline

| Time | Action | Expected System Response |
| --- | --- | --- |
| T+0s | Show dashboard Overview — all quiet, low threat | Baseline state visible |
| T+30s | Swipe unauthorized RFID card at door | Unknown Card Attempt → HIGH severity → PAC integrity bar: IDLE → HEALTHY |
| T+60s | Run python3 attack_simulator.py --type ransomware on Mac | EDR detects ransomware_behavior → CRITICAL; NDR detects lateral_movement → HIGH; Behavioral Timeline EDR line spikes |
| T+65s | Analytical Agent links PAC + Data incidents | coordinated_attack correlation fires → campus_wide_threat if multi-building; Dashboard CORRELATIONS counter increases; Kill Chain page shows new entry |
| T+70s | Orchestrator triggers 4 SOAR playbooks | Ransomware Response, Active Intrusion Response, Access Control Lockdown, Data Exfiltration Response; Dashboard SOAR page shows swimlanes; Notifications sent to CISO, IR team, DPO |
| T+90s | Show dashboard results | Overview: Threat = CRITICAL; Kill Chain: 7+ stages glowing; SOAR: Playbooks running; Forensic: New ZIP bundles; Correlations: Force graph shows cross-domain edges |

### 9.3 Human-in-the-Loop Demo

On the SOAR Response page:

Show AWAITING_APPROVAL swimlane steps

Click APPROVE on one step → watch it complete (green checkmark)

Click DECLINE on another → watch it reject (red cross)

Explain: *"High-impact actions require analyst confirmation before execution"*

## 10. System Reset Procedure

To reset the system for a fresh demo run:

# 1. Clear SOAR state
curl -s -X POST http://localhost:8007/clear_pending
curl -s -X POST http://localhost:8007/clear_executions

# 2. Stop all containers
cd ~/agents-test-main/docker
docker-compose down
docker rm -f campus-central-manager 2>/dev/null

# 3. Wait for clean shutdown
sleep 5

# 4. Restart everything
cd ~/agents-test-main
./start_hq.sh
# OR
cd ~/mass_hq_full_20260614_2144 && ./start_all.sh

# 5. Wait for grace period (120 seconds)
sleep 120

# 6. Clear startup backlog
curl -s -X POST http://localhost:8007/clear_pending
curl -s -X POST http://localhost:8007/clear_executions

## 11. Repository Structure

mass-hq/  (Arwa's GitHub — HQ subsystem)
│
├── start_hq.sh                      ← HQ-only startup script
├── start_all.sh                     ← Full system startup (auto-detects Kafka)
├── server.py                        ← Dashboard proxy (port 8080)
├── heartbeat_hq.py                  ← Real heartbeat monitor (checks /health first)
├── soc_enterprise.html              ← Main dashboard HTML entry point
│
├── src/                             ← Frontend (React + Babel, no build step)
│   ├── app.jsx                      ← Root React routing
│   ├── data.jsx                     ← Data fetching + state management
│   ├── shell.jsx                    ← Sidebar + top bar layout
│   ├── overview.jsx                 ← Overview page (honeycomb, stats)
│   ├── incidents.jsx                ← Incident table + details
│   ├── ai-intel.jsx                 ← AI Intelligence analysis
│   ├── timeline.jsx                 ← Behavioral timeline charts
│   ├── map.jsx                      ← Campus map visualization
│   ├── correlations.jsx             ← Cross-domain correlation graphs
│   ├── killchain.jsx                ← Kill chain analysis
│   ├── soar.jsx                     ← SOAR response + swimlanes
│   ├── threats.jsx                  ← Threat intelligence feed
│   ├── forensic.jsx                 ← Forensic evidence bundles
│   ├── prediction.jsx               ← ML prediction models
│   ├── compliance.jsx               ← Compliance dashboard
│   ├── report.jsx                   ← Auto-generated reports
│   ├── topology.jsx                 ← Network topology view
│   ├── twin.jsx                     ← Digital twin visualization
│   ├── roi.jsx                      ← Security ROI calculator
│   ├── marketplace.jsx              ← Agent marketplace
│   ├── tenants.jsx                  ← Multi-tenant management
│   ├── react.development.js         ← React 18.3.1 (local copy)
│   ├── react-dom.development.js     ← React DOM 18.3.1 (local copy)
│   └── babel.min.js                 ← Babel 7.29.0 (local copy)
│
├── agents/hq/                       ← HQ-level agents
│   ├── analytical_agent/main.py     ← Cross-domain correlation engine
│   ├── orchestrator_agent/main.py   ← SOAR playbook orchestrator
│   └── learning_agent/main.py       ← Adaptive ML retraining
│
├── managers/
│   ├── central_manager/main.py      ← Central incident aggregator
│   ├── iot_local_manager/           ← IoT domain local manager
│   ├── pac_local_manager/           ← PAC domain local manager
│   └── data_local_manager/          ← Data network local manager
│
├── common/                          ← Shared utilities
│   ├── kafka_client.py              ← Kafka producer/consumer + topic registry
│   ├── mqtt_client.py               ← MQTT client wrapper
│   ├── models.py                    ← Shared data models
│   └── security.py                  ← Security utilities
│
└── docker/
    ├── docker-compose.yml           ← Full stack definition (400 lines)
    ├── Dockerfile.agent             ← Single image for all Python agents
    ├── .env                         ← Environment variables (DO NOT COMMIT)
    ├── grafana/                     ← Grafana provisioning
    └── mosquitto/                   ← Mosquitto config

ti_agent/  (Standalone repo)
├── main.py                          ← Threat Intelligence agent
├── requirements.txt                 ← kafka-python, fastapi, uvicorn, requests, pydantic
└── start_ti_agent.sh                ← Startup helper

forensic_agent/  (Standalone repo)
├── main.py                          ← Forensic evidence collection agent
└── start_forensic_agent.sh          ← Startup helper

## 12. Quick Reference Tables

### 12.1 Service Ports

| Service | Port | Host | Health Check Command |
| --- | --- | --- | --- |
| Gateway Agent | 8000 | Server Room | curl http://192.168.60.10:8000/health |
| Behavioral Agent | 8001 | Server Room | curl http://192.168.60.10:8001/health |
| PAC-EDA Agent | 8002 | Server Room | curl http://192.168.60.10:8002/health |
| Credential Anomaly | 8003 | Server Room | curl http://192.168.60.10:8003/health |
| NDR Agent | 8004 | Server Room | curl http://192.168.60.10:8004/health |
| EDR Agent | 8005 | Server Room | curl http://192.168.60.10:8005/health |
| Analytical Agent | 8006 | HQ VM | curl http://192.168.12.10:8006/health |
| Orchestrator | 8007 | HQ VM | curl http://192.168.12.10:8007/health |
| Learning Agent | 8008 | HQ VM | curl http://192.168.12.10:8008/health |
| TI Agent | 8009 | HQ VM | curl http://192.168.12.10:8009/health |
| IoT Local Manager | 8010 | Server Room | curl http://192.168.60.10:8010/health |
| PAC Local Manager | 8011 | Server Room | curl http://192.168.60.10:8011/health |
| Data Local Manager | 8012 | Server Room | curl http://192.168.60.10:8012/health |
| Central Manager | 8020 | HQ VM | curl http://192.168.12.10:8020/status |
| Forensic Agent | 8021 | HQ VM | curl http://192.168.12.10:8021/health |
| Dashboard | 8080 | HQ VM | http://192.168.12.10:8080/soc_enterprise.html |
| MongoDB | 27017 | Server Room | mongosh --eval "db.adminCommand('ping')" |
| Kafka | 9092 | Server Room | kafka-broker-api-versions --bootstrap-server 192.168.60.10:9092 |
| MQTT | 1883 | Server Room | mosquitto_pub -h 192.168.60.10 -t test -m ping |
| Zookeeper | 2181 | Server Room | nc -z 192.168.60.10 2181 |

### 12.2 Machine IP Addresses

| Machine | VLAN |
| --- | --- |
| HQ VM | 12 |
| Kafka / Server Room | 60 |
| LDAP Server | 60 |
| MongoDB Server | 60 |
| Raspberry Pi | — |
| Core Switch | 40 |

### 12.3 Kafka Topics

| Topic | Publisher(s) | Subscriber(s) |
| --- | --- | --- |
| agents.heartbeats | All agents | Central Manager, Heartbeat Monitor |
| hq.incidents | Local Managers | Analytical Agent, Central Manager, Forensic Agent |
| hq.correlated | Analytical Agent | Orchestrator Agent |
| soar.commands | Orchestrator Agent | All response handlers |
| soar.responses | Response handlers | Orchestrator Agent |
| data.alerts | NDR, EDR | Data Local Manager |
| data.incidents | Data Local Manager | Central Manager |
| iot.alerts | Gateway, Behavioral | IoT Local Manager |
| iot.incidents | IoT Local Manager | Central Manager |
| iot.telemetry | Gateway Agent | Behavioral Agent, InfluxDB |
| pac.alerts | PAC-EDA, Credential Anomaly | PAC Local Manager |
| pac.events | PAC Pi (Malak) | PAC-EDA Agent |
| pac.incidents | PAC Local Manager | Central Manager |
| pac.snapshots | PAC Pi | Forensic Agent |
| learning.model_updates | Learning Agent | NDR, EDR, Behavioral |
| ti.enriched | TI Agent | Analytical Agent |
| forensic.bundles | Forensic Agent | Dashboard |

### 12.4 Docker Containers

| Container Name | Image | Service |
| --- | --- | --- |
| campus-analytical-agent | docker_analytical-agent | Analytical Agent |
| campus-orchestrator-agent | docker_orchestrator-agent | Orchestrator Agent |
| campus-learning-agent | docker_learning-agent | Learning Agent |
| campus-central-manager | docker_central-manager | Central Manager |
| campus-gateway-agent | docker_gateway-agent | Gateway Agent |
| campus-behavioral-agent | docker_behavioral-agent | Behavioral Agent |
| campus-ndr-agent | docker_ndr-agent | NDR Agent |
| campus-edr-agent | docker_edr-agent | EDR Agent |
| campus-pac-eda-agent | docker_pac-eda-agent | PAC-EDA Agent |
| campus-cred-anomaly-agent | docker_credential-anomaly-agent | Credential Anomaly Agent |
| campus-iot-local-manager | docker_iot-local-manager | IoT Local Manager |
| campus-pac-local-manager | docker_pac-local-manager | PAC Local Manager |
| campus-data-local-manager | docker_data-local-manager | Data Local Manager |
| campus-kafka | confluentinc/cp-kafka:7.5.0 | Kafka Broker |
| campus-zookeeper | confluentinc/cp-zookeeper:7.5.0 | Zookeeper |
| campus-mongodb | mongo:7.0 | MongoDB |
| campus-mosquitto | eclipse-mosquitto:2 | MQTT Broker |
| campus-influxdb | influxdb:2.7 | InfluxDB |
| campus-postgres | postgres:16-alpine | PostgreSQL |
| campus-grafana | grafana/grafana:10.3.1 | Grafana |

## 13. Troubleshooting

### 13.1 Dashboard Shows 0/18 Agents

Diagnosis:

python3 -c "
from confluent_kafka import Consumer
import json, time

c = Consumer({
    'bootstrap.servers': '192.168.60.10:9092',
    'group.id': 'debug-' + str(int(time.time())),
    'auto.offset.reset': 'latest'
})
c.subscribe(['agents.heartbeats'])

start = time.time()
seen = set()
while time.time() - start < 30:
    msg = c.poll(1.0)
    if msg and not msg.error():
        d = json.loads(msg.value().decode())
        seen.add(d.get('agent_id'))

c.close()
print('Agents on Kafka:', sorted(seen))
"

| Result | Likely Cause | Fix |
| --- | --- | --- |
| Agents appear on Kafka but not on dashboard | Central Manager is down or not consuming | Restart Central Manager container |
| No agents appear | Agents are not running or cannot reach Kafka | Check agent logs; verify Kafka connectivity |

### 13.2 Central Manager ContainerConfig Error

Cause: A known bug in docker-compose v1.29.2 with the Central Manager's container config.

Fix: Use direct docker run instead of docker-compose for the Central Manager:

docker rm -f campus-central-manager 2>/dev/null

docker run -d --name campus-central-manager \
  --network campus-mass-network \
  -p 8020:8020 \
  -e KAFKA_BOOTSTRAP=192.168.60.10:9092 \
  -e HEALTH_PORT=8020 \
  docker_central-manager python -m managers.central_manager.main

### 13.3 Dashboard Shows "Unable to Connect"

Fix:

# Check if server.py is running
pgrep -a python3 | grep server

# Kill and restart
pkill -f "server.py"
fuser -k 8080/tcp 2>/dev/null
sleep 2
cd ~/agents-test-main && python3 server.py &

### 13.4 PAC RFID Not Reading

Fix (Raspberry Pi 5):

source ~/myenv/bin/activate

# Remove the conflicting RPi.GPIO
pip uninstall RPi.GPIO

# Install the correct GPIO shim for Pi 5
pip install rpi-lgpio

# Set the chip identifier
export LG_CHIP=4

# Restart the PAC system
python3 /home/pi/mass_pac_final.py

### 13.5 Camera Not Working on Pi

Cause: OpenCV version mismatch between system Python and virtual environment.

| Environment | OpenCV Version | Camera Status |
| --- | --- | --- |
| System Python | 4.13.0 | Works |
| myenv (venv) | 4.8.0 | Broken |

Fix: Always run camera-related scripts (e.g., enroll_faces.py) with system Python:

# Wrong:
source ~/myenv/bin/activate
python3 enroll_faces.py        # ← Uses opencv 4.8.0 — BROKEN

# Correct:
deactivate                      # ← Exit virtual environment
python3 /home/pi/enroll_faces.py  # ← Uses system opencv 4.13.0

### 13.6 Pending Approvals Flooding (500+)

Cause: Kafka's historical message backlog triggers many SOAR playbooks immediately on startup.

Fix: Wait for the 120-second grace period, then clear:

sleep 120
curl -s -X POST http://localhost:8007/clear_pending
curl -s -X POST http://localhost:8007/clear_executions

### 13.7 Incident Timestamps Show Negative Time

Cause: Clock skew between machines (some machines are a few seconds ahead of the HQ VM).

Immediate workaround: Already handled in the dashboard code — negative timestamps display as "just now".

Permanent fix: Synchronize all machines to the same NTP server:

# On all machines:
sudo timedatectl set-ntp true
sudo systemctl restart systemd-timesyncd

# Verify
timedatectl status

### 13.8 Kafka Connection Refused

Diagnosis:

# Test from the failing machine
nc -z -w3 192.168.60.10 9092 && echo "Kafka reachable" || echo "Kafka unreachable"

| Scenario | Fix |
| --- | --- |
| Kafka is not started | SSH to server and start Kafka (see Section 4.4) |
| Firewall blocking | sudo ufw allow 9092/tcp on the server |
| Wrong advertised.listeners | Edit server.properties to use the correct campus IP |
| Docker network issue | Agents inside Docker need 172.18.0.1:9092 (Docker host gateway), not localhost:9092 |

### 13.9 Docker Compose "Network Not Found"

Fix:

# Create the network manually
docker network create campus-mass-network

# Then retry
docker-compose up -d

### 13.10 Agent Code Hotfix (Without Rebuilding Images)

If you need to patch agent code without a full image rebuild:

# Copy the fixed file into the running container
docker cp ~/agents-test-main/agents/hq/analytical_agent/main.py \
  campus-analytical-agent:/app/agents/hq/analytical_agent/main.py

# Restart the container to pick up changes
docker restart campus-analytical-agent